ORIGINAL ARTICLE

=

The Effects of Porestructure on Sonic Velocity in Carbonates: Examples from Gunung Rapat Limestone, Ipoh

Muhammad Hafiz Izuddin · Chow Weng Sum · Khor Wei Chung

Abstract

Correlation between rock physical properties (eg: porosity, permeability and sonic velocity) are not easy to be established. The correlation trends are governed by the pore type, size, and geometry. The preliminary results show a large scattering distributing in the correlations. The scattered correlations are may due to the different types of microporosity, pore geometry and micrite microtextures of the carbonate rocks.

Keywords – Rock Physics, Carbonate, Dolomite, Micropores, Sonic Velocity & Gunung Rapat Limestone

Introduction

It is estimated that more than 60% of the world’s oil and 40% of the world’s gas reserves are held in carbonate reservoirs. Carbonate is predominantly composed of calcite of organic, chemical or detrital origin (Blyth & Freitas, 2010). Carbonate rocks are also heterogeneous than clastic rocksand thus may contain different types of porosity (Pourmohammadi et al, 2007).  In the carbonate formations, the determination of the type and value of both primary and secondary porosity has a significance influence in giving the correct prediction of permeability and evaluation of hydrocarbon reserves (Kazatchenko et al., 2003).

Correlations between porosity and other rock properties of carbonates are not easy to be established. The heterogeneity of the pore types and grains create a large scatter in the relationship. Many studies have shown that sonic velocity and permeanbility of the

Figure 1: Location of Gunung Rapat in Peninsula Malaysia (modified from Memoir of the Geological Survey of West Malaysia, 1967).

carbonate depends not only on porosity distribution, but the pore geometry as well (Anselmetti & Eberli, 1993). Though numerous

Figure 2: XRD result of Sample 3 and Sample 1. The spikes show the presence of Calcium Carbonate, CaC03 and Magnesium Calcite, MgO·O3·CaO.

studies have been done on rock properties of Gunung Rapat carbonates (Figure 1), but quantifying the correlations between these rock properties (eg: sonic velocity to permeability) are still ambiguous (Habibur, 2011; Baechle et al., 2006). Thus, this paper will establish the correlations of porosity and other rock propertiesand determine the elements affecting the variations.

Aims

The objectives of this paper are: -

- To establish correlations between rock physical properties (eg: porosity and permeability), sonic velocity and sedimentological/ diagenetic characteristic of carbonates from Gunung Rapat, Ipoh.

- To investigate the inverse yet scattered relationship between porosity and sonic velocity of the carbonates.

- To study the relationship between sonic velocity and permeability of the carbonates.

- To confirm the direct trend in the correlation of porosity and permeability, and,

- To investigate the factors affecting the amount porosity in the carbonates (eg: dolomitization, grains size and distribution).

Methodology

This paper will present the analyses done on 20 core samples from Gunung Rapat, Ipoh. The cores are 1 inch in diameter and 1.5 inch long. Amongst the analyses done are the sonic velocity, porosity and permeability (through Helium Porosimeter and Mercury Porosimeter Pascal 140 & 240), XRD (X-Ray Diffraction) and SEM (Scanning Electron Microscope).

The classification of porosity uses the scheme proposed by Eberli (2003). This paper does not apply the classification scheme by Choquette & Pray (1970) because the porosity of the samples is minute and not visible through polarized microscope. Dunham’s classification (1962) of carbonate rocks is used for identification through thin sections. The classification of microporosity is defined by Habibur’s Scheme (2011) – very fine micropores (0.1 – 2 μm), fine micropores (2 – 4 μm),

Figure 3: Graphs illustrating the correlation between sonic velocity, porosity and permeability. A) Graph of P-wave velocity, m/s, versus porosity, %, with a correlation factor, R2, of 0.1151. B) Graph of S-wave velocity, m/s, versus porosity, %, with a correlation factor, R2, of 0.2686. C) Graph of P-wave velocity, m/s, versus permeability, mD, with a correlation factor, R2, of 0.0155. D) Graph of S-wave velocity, m/s, versus permeability, mD, with a correlation factor, R2, of 0.0101. E) Graph of P-wave velocity, m/s, versus S-wave velocity, m/s, with a correlation factor, R2, of 0.1452. F) Graph of permeability, mD, versus porosity, %, with a correlation factor, R2, of 0.4141.

medium micropores (4 – 6 μm) and coarse micropores (6 – 10 μm).

Results and Discussion

Sonic Velocity – Porosity – Permeability

The P-wave velocity obtained ranges from 4964 m/s to 6818 m/s and the S-wave velocity ranges from 2014 m/s to 4258 m/s (Figure 3). The porosity ranges from 1.6% - 6.1% (Figure 3) while permeability ranges from 0.000135 mD (miliDarcy) to 0.610234 mD. Scattering and wide trend are obtained from the established porosity and sonic velocity (P-wave and S-wave velocity) graphs. The correlations of porosity-Pwave velocity and porosity-S-wave velocity show an inverse relationship where the amount or percentage of the porosity increases as sonic velocity decreases. This inverse trend is predictable from the amount and distribution of the pore space (Blaechle et al, 2004). Theoretically, more pore space will produce a lower reading in sonic velocity. The large scattering trend in correlation is due to the heterogeneity of the pore type, pore shape and geometry (Figure 3)(Eberli et al, 2003). These scattering trends are also reported in Eberli et al. (2003) and Wang et al. (1991). It is also suggested that other than the amount of porosity, the other intrinsic properties of the rocks (eg: pore geometry and mineral composition) also affect the sonic velocity values (Wang et al, 2004).  The characteristics of carbonate sediments that are prone to rapid and pervasive diagenetic alterations (eg: continuous cementation and dissolution processes) change the mineralogy and pore structure of the rocks. These modifications affect the sonic velocity of the rocks, thus resulting in a dynamic relationship between porosity and sonic velocity (Blaechle et al., 2008). In order to have a better correlation, the classification between macroporosity and microporosity is needed (Habibur et al., 2011). The sonic velocity of a carbonate rock will be lower when the proportion of microporosity is higher (Habibur et al., 2011).

Scanning Electron Microscope (SEM)

The microporosity in carbonate rocks is affected by microtextures of micrite and not the type of micropores (Figure 4). In standard cases, fine micropores will be low in porosity. However, in this paper, fine micropores can have either low or good porosity depending on their occurrence in certain types of micrite microtexture (Sample B and C in Figure 4). High porosity rock could have faster and higher sonic velocity value due to high amount of fine micropores (Sample B in Figure 4). The micropores are too discrete to act as a porous medium.

Permeability of the carbonate rocks is also governed by the occurrence and distribution of micrite microtextures. The micropores that occur in microrhombic and polyhedral micrites tend to have better connectivity (Sample F and H in Figure 4). Occasionally, these micrites contain lower amount of microporosity due to the textural arrangements and have good inter-connectivity between the micropores (Table 1).

| Microtexture | Micropores | Fluid Flow |
| --- | --- | --- |
| Subrounded Micrites | Very fine, fine, medium | Poor – Moderate |
| Microrhombic & Polyhedral Micrites | Fine, medium, coarse | Poor - Moderate |
| Compact Anhedral Micrites | Very fine | Very poor |

Table 1: The relative contribution of different pore types of micrite microtextures on fluid flow.

G) Sample 5 - Crystalline dolomite - Very fine micropores with low porosity and poor interconnection in compact anhedral micrites. H) Sample 9 - Packestone - Coarse micropores with good porosity and moderate interconnection in microrhombic and polydegral micrites. I) Sample 14 - Crystalline dolomite - Very fine micropores with good porosity but poor interconnection in subrounded micrites.

Figure 4: SEM images on sample 15, 16, 18, 6, 4, 8, 5, 9 and 14. A) Sample 15 - Grainstone - Very fine micropores with low porosity and poor interconnection in subrounded micrites. B) Sample 16 - Packestone - micropores with good porosity and moderate interconnection in subrounded micrites. C) Sample 18 - Grainstone - Fine micropores with low porosity and poor interconnection in microrhombic and polyhedral micrites. D) Sample 6 - Crystalline dolomite - Very fine micropores with low porosity and very poor interconnection in compact anhedral micrites. E) Sample 4 - Packestone - Medium micropores with low porosity and poor interconnection in subrounded micrites. F) Sample 8 - Grainstone - Medium micropores with low porosity but moderate interconnection in microrhombic polyhedral micrites.

Conclusion

A general inverse trend in the correlation of porosity-sonic velocity is deduced from the scatted distribution of the graphs (Figure 3). The sonic velocity of the carbonate not only influenced by the amount of porosity, but also by the crystallometry and morphometry of the micrite microtextures. The correlation of porosity-permeability has a direct trend. Crystallometry and morphometry of micrite microtextures govern the trend of this correlation.

Reference

Anselmetti., F., S. & Eberli, G., P., 1993. “Controls on Sonic Velocity in Carbonates”. Basel: PAGEOPH Vol 141, No: 2/3/4

Baechele, G., T., Eberli, G., P., Boyd, A., DeGrange, J., M. & Al-Kharusi, L., 2008. “Oolmoldic Carbonates: Pore Structure and Fluid Effects on Sonic Velocity”. Las Vegas: SEG Annual Meeting.

Baechle., G., T., Weger, R., Eberli., G., P. & Colpaert, A., 2006. “Pore Size and Pore Type Effects on Velocity – Implicaiton for Carbonate Rock Physic Models”. Berge, Norway: Workshop “Sound of Geology”.

Baechle., G., T., Weger, R., Eberli., G., P. & Massaferro, J., L., 2004. “The role of Macroporosity and Microporosity in Constraining Uncertainties and in relating Velocity to Permeability in Carbonate Rocks, Denver, Colorado: SEG International Exposition and 74th Annual Meeting.

Blyth, F., G., H., & Freitas D., M., H., 2010. “A Geology for Engineers”. 7th Edition, Great Britain, Butterworth Heinemann.

Eberli, G., P., Baechle, G., T., Anselmetti, F., S. & Incze, M., L., 2003. “Factors Controlling Elastic Properties in Carbonate Sediments and Rocks”. D. Clark Edition. The Leading Edge, Society of Exploration Geophysicists.

Eberli, G., P., Baechle, G., T., Weger, R. & Massaferro, J., L., 2004. “Quantitative Discrimination of Effective Porosity using Digital Image Analysis: Implications for Porosity-Permeability Transforms”. Paris: 66th EAGE Conference.

Habibur, R., Pierson, B., J. & Wan Ismail, W., Y., 2011. “Quantification of Microporosity and its Effects on Permeability and Sonic Velocity in Miocene Carbonate Reservoirs”. Offshore Sarawak, Malaysia. Universiti Teknologi PETRONAS, Kuala Lumpur: Seacarl.

Habibur, R., Pierson, B., J. & Wan Ismail, W., Y., 2011. “A New Approach to Classify Microporosity in Miocene Carbonate Reservoirs of Offshore Sarawak, Malaysia”. Richardson, Texas: IPTC 14583

Habibur, R., Pierson, B., J. & Wan Ismail, W., Y., 2011. “Classification of Microporosity in Carbonates Examples from Miocene Carbonate Reservoirs of Central Luconia, Offshore Sarawak, Malaysia”. Richardson, Texas: IPTC 14583

Kazatchenko, E., Markov, M. & Moutsavov, A., 2003. “Determination of Primary and Secondary Porosity in Carbonate Formations Using Acoustic Data”. Richardson, Texas: SPE 84209

Pourmohammadi, S., Hetland, S., Spildo., K. & Skauge, A., 2007. “Fluid Flow Properties of Different Carbonate Classes”. Richardson, Texas: SPE 111433

Wang, Z., Hirsche, W., K. & Sedgwick, G., 1991. “Seismic Velocites in Carbonate Rocks”. Journal of Canadian Petroleum Technology.