Spatial variation of groundwater quality parameters: A case study from a semiarid region, of Iran

A. Taheri Tizro1*, K.S. Voudouris2, S. Vahedi3

1,3 Dept. of Water Engineering, College of Agriculture, Bu-Ali Sina University, Iran

2Lab. of Engineering Geology & Hydrogeology, Dept. of Geology, Aristotle University, Egnatia str., 54124 Thessaloniki, Greece, E-mail: kvoudour@geo.auth.gr

E-mails: ttizro@yahoo.com, ttizro@basu.ac.ir, kvoudour@geo.auth.gr

*Corresponding Author

Abstract

The Zarin Abad plain is located in Zanjan province in the western part of Iran. Water needs of the plain are covered by numerous wells, which exploit the aquifer systems developed within alluvial deposits of the plain. In this study, spatial analysis was used to interpret some of the chemical parameters of groundwater samples collected from the unconfined aquifer. For this reason, groundwater samples (61) were collected from shallow and deep groundwater monitoring wells to investigate the characteristics of groundwater quality. Furthermore, detailed local geological, geophysical and hydrogeological investigations were carried out to delineate the architecture of different subsurface geological horizons. Maps showing the geographical distribution of hydrochemical parameters were prepared and the impact of various geological formations on the groundwater quality in the region is considered. High values of EC value in the western parts of the plain could be associated with the lithological formations composed of marls and evaporates. The salinity may be attributed to long residence time of water and the dissolution of minerals. Based on SAR values, it is concluded that the majority of groundwater samples is relatively suitable for irrigation use.

Analysis of the spatial coherence of the variables from each other was performed using selected models and the Kriging, Co-Kriging and Inverse Weighted Distance (IWD) methods were ultimately used to decipher the spatial variation of the parameters. The results obtained through these methods were compared by root-mean-square error (RMSE) and mean absolute error (MAE). It was found that the Co-Kriging model is the most optimal technique, for studying the spatial distribution in groundwater quality parameters.

Keywords: Groundwater quality, Spatial distribution, Geostatistical analysis, GIS, Kriging, Iran

Introduction

Groundwater quality is influenced by the geological formation and anthropogenic activities, e.g. changes in land use, urbanization, intensive irrigated agriculture, mine activities, disposal of untreated waste effluent in river, lack of rational management, etc (Voudouris 2009). It is pointed out that, groundwater contamination may cause various diseases and other problems on consumption (Fetter 1993).

To establish hydrochemical classes of groundwater in a study, it is often necessary to use different approaches and methods for the graphical spatial and temporal representation of the relevant data. Multivariate data analysis has previously been used by researchers in other studies as part of the interpretation of groundwater quality (Davis 1986; Voudouris et al. 1997; Voudouris et al. 2000; Lambrakis et al. 2004; Baalousha 2010). Geostatistcial methods are increasingly applied because it allows one to capitalize on the spatial relationship between neighbouring values of the hydrochemical parameters, in order to predict attribute values at unsampled sites (Goovaerts 2000; Nas 2009). The representation of spatial distribution of groundwater quality is an important task, and special attention is necessary in studies of the hydrogeochemistry (Bárdossy and Kundzewicz 1990).

One of the non-classical methods which has been used in determining the spatial distribution of groundwater quality parameters is the geostatistical appoach. Geostatistical methods, which are considered a powerful tool for interpolation, have been applied in different branches of science, such as Earth science, Hydrogeology, Soil science, Mining and Hydrometeorology (Robinson and Metternicht 2006; Stafanoni and Hernandez 2006; Xu at al. 2006). Recently, geostatistical tools have also been applied in the modelling of the spatial distribution in many disciplines. These tools are increasingly coupled with GIS capabilities for applications that characterise the space structure (semivariogram analysis), spatially interpolating scattered measurements to create spatially exhaustive layers of measured parameters (Goovaerts 2000; Mehrjardi et al. 2008). Kriging is regarded as a best linear unbiased estimation (BLUE). Commonly used methods applied in spatial statistics include: Ordinary Kriging, Co-Kriging and Inverse Weighted Distance (IWD). Many studies have been conducted by different researchers in recent years with the aim of modelling water quality variations with respect to depth and lithological formations (Bárdossy and Kundzewicz 1990; Istok and Cooper 1998; Gauss et al. 2003; Theodosiou and Latinopoulos 2006; Feng-Guang et al. 2008; Sun et al. 2009; Rezai et al. 2010).

Geostatistical analysis provides a suite of statistical models and tools for spatial data exploration and surface generation of groundwater quality (Sarukkalige 2012). The present study investigated the spatial distribution of the water quality parameters of the Zarin Abad plain (Zanjan province, Iran). Geostatistical methods were applied in order to determine the most suitable method that can be used to develop spatial variability maps and study the variability of the groundwater quality parameters. First, the hydrogeological setting of the study area was investigated using drilling, pumping tests and geophysical data. Secondly, the general characteristics of groundwater quality data and the accuracy of different interpolation methods (Ordinary Kringing, Co-Kringing, IWD) were examined.

Description of the study area

The Zarin Abad plain covers an area of 5107 km2 and is located in the western part of Iran (Fig. 1). The study area lies between latitude 35°10´ and 36°13´ north and longitude 47°56´ and 48°57´ east. Twenty nine (29) tributaries flow into the Perennial River in Zarin Abad plain, and these tributaries flow following periods of high rainfall. The river beds are filled with silt, sand and gravel in the northwestern part. In a large part of the study area irrigated agriculture was applied.

Climate and rainfall

The study area has a temperate climate, with cold winters and moderate summers. The average minimum and maximum temperature are recorded in January (-3.6 oC) and July (21.8oC), respectively. According to the classification of De Martin the climatic type is semi-arid. The range of annual rainfall over a recent 20 year period (1985–2005) was 272–373 mm (Anonymous 2007). Generally, the higher the topographic elevation the higher the rainfall. Rainfall mainly occurs during the wet period, with its maximum in November and December. July and August are the months of lowest rainfall and form the dry period.

Geological setting

According to Stocklin (1986) the study area belongs to the central Iran tectonic zone. The area has been mapped by the Geological Survey of Iran at 1:1,250,000 scale, and the geology has been described by Boulourchi (1979), Braud (1970) and Darvish Zadeh (2002). The oldest geological series of rocks in the plain are slate and schistose rocks, which are partly metamorphosed and belong to Upper Triassic-Jurassic sequences. The crystallisation of limestone is observed in-between these rocks. Rocks with low metamorphic formations in the area are Nayeen and Shemshak formations, and outcrop in the northeast and southeast parts of the plain. Conglomerate and sandstone with a limestone base of Cretaceous rocks belonging to the Albian along with andesite-basalt, Albian shales and shales belonging to the Upper Cretaceous and are exposed in the southern parts and have all been metamorphosed. The Cretaceous exposures form slopes against Jurassic rocks (Fig. 1).

The Tertiary rocks which lie on older rocks with unconformities are composed of conglomerate and red sandstone formations, named Fajan formations. These rocks are Eocene in age. Volcanic rocks such as andesite-dacite-rhyolite are exposed to view in the southern parts of the area, along with green marl and sandstone. The Miocene rocks (Upper Red Formation) of clastic sediments, including marls, red sandstone and conglomerates, are mixed with gypsum and salt layers and exposed to view in northern parts. The Pliocene sediments include clay and sand with horizontal bedding and are deposited in the central and western parts of the basin, which lies on the Upper Red Formation. Quaternary deposits (recent alluvium) dominate in a large part of the plain.

Hydrogeology

The complicated geological formation has created a special condition from a hydrogeochemical point of view. The main aquifer system is developed within the alluvial deposits. The depth to the water table is highly variable; it is relatively shallow (6 m below ground surface) in the central part near Zarin Abad River and as deep as 60 m below ground surface or more in the eastern parts. The water table is highest in May or April and lowest in October or September.

Groundwater flows from the recharge area of the upper hills of east and northeast and slopes toward the central part of the plain and ultimately discharges to the Zarin Abad River. Transmissivity is in the order of 30 m2/day around the periphery, and up to 900 m2/day in the southern part of the basin, as deduced from pumping test analyses carried out in the frame of previous hydrogeological investigation (Tizro and Signal 1993). Lower transmissivity occurs in the northwestern part (Fig. 2). The higher values of transmissivity in the southern part of the plain could be attributed to greater depth of aquifer in these parts. Estimates for the yield of the boreholes range from about 30 and 120 m3/h. The limited availability of lithological data meant it was difficult to study the subsurface geology and posed severe constraints in compiling a holistic picture. These shortcomings were, however, overcome by the use of geophysical data.

Geophysical measurements and interpretation

A geophysical investigation with a Schlumberger configuration in the area was conducted by Ministry of Power (Anonymous 2007). For many locations new data were generated in the field. The apparent resistivity data obtained for different values of AB/2 have been processed. These results were subsequently used to obtain a realistic picture of the geological framework (Tizro 2002; Tizro and Singhal 1993; Tizro et al. 2010). Quantitative (indirect and direct) and geologic methods have been applied in the interpretation. Bhattacharya and Patra (1968) and Griffith and King (1965) described the techniques used in the indirect interpretation. Comprehensive accounts of various approaches to the automatic interpretation of resistivity data have been given by Zohdy et al. (1974) and Niwas and Singhal (1983). The automatic interpretation of sounding data (Zohdy 1989), which produces the interpreted depths and resistivities respectively, has been utilised to obtain the true resistivity values and the corresponding layer thicknesses.

The data was compared with the lithologs of boreholes located in the vicinity of the corresponding soundings. Resistivities for sandy horizons vary from 35–100 Ω-m, whereas predominantly clay and silt zones show ranges of much lower resistivity (2 to 7 Ω-m). The rock formations exhibit wider ranges of resistivity (150–400 Ω-m). Low values of resistivity for these formations imply a friable and weathered nature.

A fence diagram (Fig. 3), constructed from the resistivity data, illustrates the stratigraphic framework of the basin in quasi-three-dimensional form. Geological and geophysical studies have revealed the existence of a deeper aquifer overlain by thick, relatively impermeable Pleistocene beds in the western parts of the basin. Only limited numbers of deep wells (depth >60 m) have been drilled in the area. In the southern part of the basin, weathered and fractured rocks are likely to form aquifers, as is clear from the subsurface layers along the eastern parts of the plains composed of fine layers of clay and exhibiting very low resistivity. The depth of these layers reaches up to 300 metres. A large part of the southeastern plain is composed of sands and gravels. In this part the main and more productive aquifer system is developed.

Groundwater quality monitoring

To evaluate the groundwater quality characteristics of the study area, a program of groundwater quality monitoring was developed in order to bring out the relevant parameters of water quality for the whole area with a network of optimally selected observation wells, spring and qanats keeping in view the overall distributions of alluvium and consolidated geological formations. It is pointed out that qanat is a system of water supply consisting of an underground tunnel connected to the surface by a series of wells which uses gravity to bring water from the water table of the higher elevation to the surface of the lower elevation lands (Voudouris et al. 2013).

In the frame of hydrochemical investigation sixty one (61) representative water samples were collected from available points during the hydrological year 2007/2008 (one round of sampling), according to the sampling protocol described by Claasen (1982) as follows: 32 wells (shallow and deep), 21 springs and 8 qanats. Shallow and deep wells were sampled at the same time. Deep wells are considered the wells with more than 60 m depth in this study. The majority of the wells were constructed by stainless steel and a small number from PVC. The sampling sites are shown in Figure 1. The water temperature (oC) at the head of the wells, the Electrical Conductivity (EC), Total Dissolved Solids (TDS) concentration and the pH values of 62 samples were measured in situ, using a portable  multimeter (MM-40, Crison). After pumping of production wells for about 30 minutes, water samples were collected in polyethylene bottles (1.5 L).

The samples were analysed with the procedures described by standard methods (AWWA, 2010) in the laboratory of the Zanjan Regional Water Centre (ZRWC) for major ions Ca2+, Mg2+, Na+, K+, Cl-, NO3-, SO42-, HCO3- and Total Hardness (TH).  Na+, K+ cations were measured by flame photometry. Ca2+ and TH were determined by the EDTA titrimetric method. Anions NO3- and SO42- were determined by using the Hach spectrophotometer. HCO3- was measured by 0.02 N H2SO4 titration method immediately after sample collection. Finally, Cl- was measured by titrimetric method using AgNO3 and Mg2+ was determined from Total Hardness.

Sodium Adsorption Ratio (SAR) and Sodium content or percentage (%Na) were calculated using the results of chemical analyses. SAR is defined by the following equation (1), where the ionic concentrations are expressed in meq/L (Todd and Mays 2005):

SAR=                                       (1)

Sodium content (%Na) is usually expressed in terms of percent sodium, and is calculated by equation (2), where all the concentrations are expressed in meq/L (Fetter 1993; Todd and Mays 2005):

%Na=   	                           (2)

Sodium Adsorption Ratio (SAR) and Sodium content (%Na) are useful indicators for irrigational suitability of groundwater in the study area.

The overall precision of the analyses is within ±5%, as indicated from the ionic balance (equation 3), using the major ions (Na+, K+, Ca2+, Mg2+, SO42-, HCO3-, NO3-, Cl-). The results of the analyses are shown in Table 1.

Error percent (%) = x100                             (3)

Geostatistical methods

Geostatistics is based on the theory of the study of regionalized variables (Goovaerts 2000). For the purpose of evaluating spatial distribution in groundwater parameters from each other, the variable mode was used. In general, the purpose of calculating the variable mode is to estimate changes in variables with respect to temporal and spatial variations. To calculate this variability, the total squared difference between pairs a distance h from each other is determined and plotted against h, in a semivariogram, as given in equation (4) (Lopez-Granados et al. 2002).

γ(h) = ½ Var [Z(x)-Z(x+h)]		                                    (4)

Where: h denotes the distance (lag) between pairs of sampling points

Var is the variance

Z(x) is the observed value of one parameter at point xi

Z(x+h) is the value of parameter at point xi+h.

The variogram is a quantitative descriptive statistics that can be graphically represented in a manner which characterizes the spatial continuity (i.e. roughness) of a data set. In order to interpolate with different methods, a theoretical model must be fitted to the experimental data of the semivariogram. By fitting a theoretical model to the value of the empirical model and by applying linear and nonlinear models, unknown variables can be estimated (Lopez-Granados et al. 2002). In this study, three types of model (spherical, exponential  and linear) were used to determine the best model variable mode. Each hydrochemical parameter was analysed under these aforementioned semivariogram models.

For the calculation of both the experimental and theoretical semivariograms the software package SURFER (Golden Software Inc., 1999) was used. The range, sill and Nugget effect are the most important parameters of each variable mode (Kitanidis 1997). The range is the distance that the variable mode approaches the fixed limit, and is close to horizontal stage. The sill is a fixed value in which the variable mode reaches the range. In general, the range can be assumed to be equal to the total variance of the samples (C+Co). The Nugget effect is the value of the mode variable in the origin of the coordinates for h=0.

Exploratory data analysis (EDA) was carried out in the primary stage to observe the normality and trend of the data. The generation of the semivariograms and the kriged maps were performed and the collected data were subjected to EDA through data transformations and outlier removal techniques, using the Arc Map module of ArcGIS (Version 9.3).

In geostatistical methods, the estimated values are obtained from total results of  multiplication of a weighting coefficient in values of observational points (Schroder 2006). The general interpolation is given in equation (5) (Feng_Guang et al. 2006):

Xi(P)=i=1nλiXi(O)	(5)

Where:  Xi(P) is an estimated value, Xi(O) the measured value, n is the number of data and λi is the weighted measured parameter of X in point i.

In this study, Ordinary Kriging, Co-Kriging and Inverse Weighted Distance (IWD) methods were applied.

Ordinary Kriging

Ordinary Kriging is an unbiased linear method which is based on the logic of a moving average (Krige 1978; Delhomme 1978; Webster and Oliver 2001). The most obvious advantage of Kriging algorithm is to calculate the error associated with each estimate. In this method, the results should be unbiased weighting coefficients which are determined so that their sum is one (equation 4. In normal Kriging the average value is unknown but is independent of the coordinates. The Kriging estimator satisfies the following conditions (Feng-Guang et al. 2008):

i=1nλi= 1 and    σ2= E{[Ž(xo)-Z(xo)]2} min	(6)

Where λi is the Kriging weight, Ž is an estimator of unknown true value of Z(xo).

Co- Kriging

The Co-Kriging method is based on the theory of the cross variogram in which the secondary variables (auxiliary) is used. Equation (7) represents the weighted combination of the values ​​for both the primary and secondary variables in the Co-Kriging method (Mehrjardi et al. 2008):

X0(P)=i=1naiXi(O)+j=1mbjYj(O)	    	      (7)

In equation (4), X0(P) is the estimated value, Xi(O) is the original measured data values, Y is the measured secondary data values, n is the number of original data, m is the number of secondary data and weighting coefficients, and a and b are the weighted coefficients of measured parameters X and Y. The estimation of the auxiliary variable (secondary) for each parameter to be used in the Co-Kriging model, the Pearson correlation coefficients method was used by application of SPSS software. The advantages of the Co-Kriging are realized through reductions in costs or sampling efforts (Mehrjardi et al. 2008).

Inverse weighted distance (IWD)

IWD is the weighted method which is related to each of the measured data and has a direct relationship with the inverse distance of an unknown point. The weighting is controlled by the power of weights, such that if the power is greater the effect of the points to distance is greater than expected (Goovaerts 2000; Nas 2009). If the power is smaller, the uniformity of all parts is considered. The points of equal distance have equal weights. The weight coefficients can be obtained from equation (8) (Burrough and McDonnell 1998):

λ=	     	     	                                    (8)

Where:   λi  is the weight at point i

Di is the distance of point i to an unknown point

α is the power ten of the weighting.

Evaluation criteria

In order to evaluate and select the best method of interpolation coefficient, the coefficient of determination of (R2), Root Mean Square Error (RMSE) and Mean Absolute Error (MAE) were used (equations 9, 10, 11) (Kitanidis 1997; Robinson and Metternicht 2006):

R2=i=1nXiO−Xmean2−i=1nXiO−XP2i=1nXiO−Xmean2				            	(9)

RMSE=1ni=1nXiO−XP2 							          (10)

MAE=i=1nXi(O)−Xi(P)n 								          (11)

In the above equations, n is the number of data, Xi(O) is the measured value, X(mean) is the average of the measured values and X(P) is the estimated value.

Data validity was performed using Statistical Package for the Social Sciences (SPSS, version 18) to verify normal distribution data. Parameters such as pH, HCO3- and TDS have a normal distribution, but other parameters did not follow a normal distribution. Therefore, normalisation of the logarithm was used. In order to determine the spatial coherence of each of the parameters and to identify the best model variable mode, a variogram of each parameter was drawn through exponential, spherical and linear models using a    relationship (Cambardella et al. 1994). The lowest Residual Sum of Squares and spatial coherence of the best model for the variable mode of each of the parameters were determined. The relationship  is an indicator of the spatial structure of the variables used. If it is more than 0.75 a weak spatial correlation exists and if it is less than 0.25 it is a parameter with a high spatial correlation (Robinson and Metternicht 2006).

Results

Groundwater Quality

As it can be seen from Table 1, major ion concentrations in groundwater are: Ca2+ (1.47-28.4 meq/L), Mg2+ (1.01-16.48 meq/L), Na+ (0.41-23.34 meq/L), HCO3- (12.08-9.77 meq/L) and SO42- (0.37-34 meq/L). The Cl- concentration ranges from 0.16 meq/L to 1465 meq/L; the mean value is 4.67 meq/L. The fluctuation of pH between 6.85 to 7.78 shows that groundwater is slightly acid to alkaline. Electrical conductivity (EC) values range between 480 and 6580 μS/cm. Based on the mean values (Table 1), the order of major cations is: Na+>Ca2+>Mg2+ and the order of major anions is: SO42->Cl->HCO3-. A increase of salinity (TDS) is recorded along the direction of groundwater flow (down-gradient part of the plain), due to long residence time of water and the dissolution of the minerals.

Correlations between major ions were carried out using the Sprearman’s correlation analysis (Swan et al. 1995). Table 2 indicates the correlation coefficients between the major ions in the groundwater. High positive correlation (>0.70) and significant at p-level 0.05 have been obtained between: Na+ & Cl- (0.90), Na+ & SO42- (0.95), Na+ & Mg2+ (0.86), Na+ & HCO3- (0.85), Mg2+ & Cl- (0.70) and Mg2+ & SO42- (0.83). From the above analysis it is recorded a strongly correlation between ions of the some charge but different valence number, as well as a relatively strong chemical correlation between ions of the some type of charge and different valence number (Voudouris et al. 2000). The abovementioned correlation values between ions can be used to determine the common (or no common) origin of the ions in groundwater. It is pointed out that the simultaneous increase or decrease in the ions in groundwater indicates of minerals dissolution. Electrical Conductivity (EC) mainly depends on the concentration of major ions, such as Na+, Ca2+, Mg2+, Cl-, SO42- and HCO3-. Voudouris et al. (2000) applied multiple regression analysis between EC and major ions in order to determine the percentage of each ion contributing to the EC.

Geostatistical analysis

Variogram evaluation results indicate a high spatial coherence for Na, whereas EC, TDS, and HCO3– are medium and pH has a very poor spatial coherence. The characteristics of the best variable mode model, the range, sill and Nugget effect of each parameter, is given. Most parameters in the exponential model showed better results (Table 3). Theoretical models chosen for the above mentioned parameters were the exponential, except of parameter pH, which follows the linear model, as shown in Figs 4 and 5.

To estimate the spatial variation of the parameters, the geostatistical method of the Arc GIS (version 9.3) software was used. Different geostatistical methods; Kriging, Co-Kriging and IDW with powers 1, 2, 3 and 4 were selected to estimate the spatial hydrochemical parameters. For the purpose of comparing the results, the RMSE and MAE methods were applied (Table 4). The best method for estimating the TDS parameter was the Kriging. The other parameters were determined by Co-Kriging method. The spatial maps were prepared in GIS environment.

The spatial variation of pH is shown in Fig. 6. The minimum and maximum values of pH were measured in situ as 6.85 and 7.78 with a mean value 7.23 and standard deviation 0.195.  As it is mentioned before, Electrical conductivity (EC) values vary from 480 to 6580 µS/cm; the mean value is 1695 µS/cm. The spatial distribution of EC in the alluvial aquifer shows an increase in EC values towards the central and western parts (Fig. 7). This increase may be due to the presence of sodium chloride in the sediments of the lower red formations and to the lithological formations composed of marls and evaporates (halite and gypsum), respectively. As expected, the total dissolved solids (TDS) follows a similar pattern (Fig. 8), it is low in the recharge area and progressively increases towards the downward gradient part of the plain.

As is clear from the subsurface layer studies along the western parts, the plain is composed of fine layers of clay and exhibits very low resistivity (Fig. 3). The depth of these layers reaches to 300 metres. Because the recharge is very low, the low permeable sediments are not adequately flushed and can thus provide quality problems outweighing their relatively low flow volumes (Tizro 1995). The presence of sudden changes in salinity in some parts of the plain is due to the high content of fine sediments and dissolved materials.

It is found that the Sodium Adsorption Ratio (SAR) increases in the upstream parts. Most of the chemical parameters increase in the southern parts of the plain. This increase can be due to the dissolution of the minerals and the presence of farmland in the area such as presence of pesticides, herbicides and fertilizers. The spatial variation of SAR is shown in Fig. 10.

The sodium percentage (%Na) is important for irrigation because it reacts with the soil to reduce its permeability (Todd and Mays 2005). Soil containing a large proportion of sodium with carbonates as the predominant anion are termed alkali soils; those with chloride or sulphate as the predominant anion as saline soils (Tizro and Voudouris 2008). Either type of sodium saturated soil will support little or no plant growth. Based on %Na values, it is concluded that the groundwater is relatively suitable for irrigation use (%Na <35), except of the water samples from the wells numbered 1, 7, 8, 10, 12, 21, 22 and 23 (%Na >35). Fig. 9 shows the spatial variation of Na+ using the Co-Kriging method.

Conclusions-Discussion

Zarin Abad plain is located in the western part of Iran and is characterized by semiarid climate conditions. Quaternary sediments consisting of clay and sand with horizontal bedding are deposited in the central and western parts of the plain. The main aquifer systems of high transmissivity are developed within these sediments in the study area part.

In this study, 61 groundwater samples were used to estimate the spatial variation of some chemical parameters of groundwater in the Zarrin-Abad plain. The first objective was to investigate and map the groundwater quality using geostatistics. Lack of data in northern and southern parts of the study area was a significant issue and it was a reason to apply the geostatistical analysis in this investigation.

Analysis of the spatial coherence of the variables was performed using selected models and the Kriging, Co-Kriging and Inverse Weighted Distance (IWD) methods were ultimately used to decipher the spatial distribution of the parameters. The results obtained through these methods were compared by RMSE and MAE, and it is found that the Co-Kriging model is the most optimal technique, or use in studying the spatial variation in groundwater quality parameters.

The range of pH values is 6.85-7.78 with a mean value 7.23 indicating the alkaline character of groundwater. Electrical conductivity (EC) values range between 480 and 6580 μS/cm. High values of EC are recorded in the western parts of the plain. This increase can be associated with the lithological formations composed of marl and evaporates (halite and gypsum). An abnormal increase in electrical conductivity may be due to the presence of sodium chloride in the sediments of the lower red formations. The higher values of total dissolved solids (TDS) in groundwater are associated with high concentrations of all major ions.

There is wide range of values of major ions in groundwater: Ca2+, Mg2+, Na+, HCO3- and SO42-. The Cl- concentration ranges from 0.16 meq/L to 1465 meq/L; the mean value is 4.67 meq/L. High Cl- values may be attributed to the upconing or lateral movement of old saline groundwater. The salinity may be attributed to long residence time of water and the dissolution of minerals, followed by evaporation of rainfall and irrigation returns (Tizro and Voudouris 2008). After short rain events and irrigation periods, the water is consumed by evapotranspiration and salts are precipitated. During the large rainfall or irrigation these salts are dissolved and leached into the subsurface (Mahlknecht et al. 2004). The high sulphate (SO42-) concentrations in groundwater could be associated with the dissolution of the mineral pyrite (FeS2). It is pointed out that sulphur is widely distributed in both igneous and sedimentary rocks, as metallic sulphides (Hem 1985). Based on SAR values and sodium content (%Na), salinity appears to be responsible for the poor groundwater quality. Figure 9 shows the spatial variation in Na using the Co-Kriging method. The majority of groundwater samples is relatively suitable for irrigation use (sodium content %Na <35). Finally, according to the groundwater quality parameters distribution maps, using geostatistical analysis, it is concluded that the eastern part of the plain has the better quality (low EC, TDS, SAR and Na+ values).

The aforementioned geostatistical analysis and the produced maps are a useful tool for hydrogeologists and engineers to estimate water quality in areas without sampling points (e.g. in the northern part of the study area). In addition, are useful in order to protect the groundwater quality and apply a strategy of sustainable water management. The results provide important information, while the spatial distribution maps of groundwater quality could potentially be used by local authorities and decision makers in order to assess vulnerable zones and prevent further pollution of already contaminated areas. For example, prior to drilling new boreholes, groundwater quality maps produced from this investigation, should be taken into account by local authorities (Nas 2009).

Further hydrochemical studies should be carried out to investigate the impacts of land use and anthropogenic activities on groundwater quality in the study area. Factors influencing the groundwater quality should be addressed as early as possible and to kept as minimum as possible (Sarukkalige 2012). Finally, future investigations of the groundwater quality in the study area would benefit by improvement in hydrogeological and hydrochemical data monitoring.

References

Anonymous (2007) Groundwater Monitoring in Zarin Abad plain. Unpublished report. West Water Regional Company. Province of Kermanshah, Ministry of Power. I.R. Iran.

Baalousha H (2010) Assessment of a groundwater quality monitoring network using vulnerability mapping and geostatistics: A case study from Heretaunga Plains, New Zealand. Agricultural Water Management 97: 240–246.

Bárdossy A, Kundzewicz ZW (1990) Geostatistical methods for detection of outliers in groundwater quality spatial fields. Journal of Hydrology 115(1-4): 343-359.

Bhattacharya PK, Patra HP (1968) Direct current geoelectric sounding: Elsevier, Amsterdam, 139 p.

Boulourchi MH (1979) Explanatory text of Kabudar Ahang Quadrangle map. Geological and Mineral Survey of Iran Publication, Geological Quadrangle D5, 107 p.

Braud J (1970) Les formations du Zagros dans la region de Kermanshah, Iran. Bull. Soc. Geol. France (7), XIII, No 3-4: 416-419 (in French).

Burrough PA, McDonnell RA (1998) Creating continuous surfaces from point data. In: Burrough, P.A., Goodchild, M.F., McDonnell, R.A., Switzer, P., Worboys, M. (Eds), Principles of Geographic Information Systems. Oxford University Press, Oxford, UK.

Cambardella CA, Moorman TB, Novak JM, Parkin TB, Karlen DL, Turco RF, Konopka AE (1994) Field- scale variability of soil properties in central Iowa soils. Soil Science Soc. Am. 58: 1501-1511.

Claasen HC (1982) Guidelines and techniques to obtain groundwater Quality samples. U.S. Geological Survey. Open report 82-1024: 54.

Darvish Zadeh (2002) Geology of Iran. Amirkabir publication, Tehran, Iran, 600 p.

Davis J.C. 1986. Statistics and data analysis in Geology. John Wiley & Sons Inc., New York, 647 p.

Delhomme JP (1978) Kriging in Hyrdosciences. Adv. Water Res. 1: 251-266.

Feng-Guang Y, Shu-You C, Xing-Nian L, Ke-Jun Y (2008) Design of groundwater level monitoring network with ordinary Kriging. J. Hydrodyn. 20: 339–346.

Fetter CW (1993) Contaminant Hydrogeology, Waveland press, Inc. Long Grove, Illinois, 500 p.

Gaus I, Kinniburgh DG, Talbot JC, Webster R (2003) Geostatistical analysis of arsenic concentration in groundwater in Bangladesh using disjunctive Kriging.  Environmental Geology 44: 939-948.

Goovaerts P (2000) Geostatistical approaches for incorporating elevation into the spatial interpolation of rainfall. Journal of Hydrology 228: 113-129.

Griffith DH, King E (1965) Applied Geophysics for Engineers and Geologists: Paragon press, London, 223 p.

Hem JD (1985) Study and interpretation chemical characteristics of natural water.  U.S. Geol. Survey Water Supply, Paper No 2254.

Istok JD, Cooper RM (1998) Geostatistics applied to groundwater pollution. III: Global Estimates. Journal of Environmental Engineering 114(4), 915-928.

Kitanidis PK (1997) Introduction to Geostatistics. Cambridge University Press.

Krige D (1978) Lognormal DeWijsian Geostatistics for ore evaluation, South African I.M.M. 55 p.

Lambrakis N, Antonakos A, Panagopoulos G (2004) The use of multicomponent statistical analysis in hydrogeological environmental research. Water Research 38: 1862-1872.

Lopez-Granados F, Jurado-Exposito M Atenciano S, Garcia-Ferrer A, Sanchez de la Orden M, Garcia-Torres L (2002) Spatial variability of agricultural soil parameters in southern Spain. Plant and Soil 246: 97–105.

Mahlknecht J, Schneider JF, Merkel BJ, Navarro de Leon I, Bernasconi SM (2004) Groundwater recharge in a sedimentary basin in semi-arid Mexico. Hydrogeology Journal 12: 511-530.

Mehrjardi TR, Jahromi ZM, Mahmodi Sh, Heidari A (2008) Spatial distribution of groundwater quality with geostatistics. Case study: Yazd-Ardakan plain. World Applied Sciences Journal 4(1): 9-17.

Nas B (2009) Geostatistical approach to assessment of spatial distribution of groundwater quality. Polish Journal of Environmental Studies, Vol. 18, No 6:1073-1082.

Niwas S, Singhal DC (1983) Aquifer transmissivity of porous media from resistivity data: Journal Hydrology, Vol. 82, No. 1-2: 143-153.

Rezai M, Davatgar N, Tajdari K, Abolpour B (2010) Investigation the spatial variability of some important groundwater quality factors in Guilan, Iran. Journal of Water and Soil 24: 932-941.

Robinson TP, Metternicht G (2006) Testing the performance of spatial interpolation techniques for mapping soil properties. Computer and Electronics in Agriculture 50: 97-108.

Sarukkalige R (2012) Geostatistical analysis of groundwater quality in Western Australia. IRACST- Engineering Science and Technology: An International Journal (ESTIJ), Vol. 2, No 4: 790-794.

Schroder W (2006) GIS, geostatistics, metadata banking, and tree-based models for data analysis and mapping in environmental monitoring and epidemiology. International Journal of Medical Microbiology 296 S1: 23–36.

Stefanoni LH, Hernandez RP (2006) Mapping the spatial variability of plant diversity in a tropical forest: Comparison of spatial interpolation methods. Environmental Monitoring and Assessment 117: 307-334.

Stocklin J (1986) Structural history and tectonics of Iran. A review. Am. Assoc. Petroleum Geol. Bull., Vol. 52, No 7: 1229-1258.

Sun Y, Shaozhong K, Li F, Zhang L (2009) Comparison of interpolation methods for depth to groundwater and its temporal and spatial variations in the Minqin oasis of northwest China. Environmental Modelling & Software 24: 1163–1170.

Swan ARH, Sandilands M, McCabe P (1995) Introduction to Geological Data Analysis. Blackwell Science, 446 p.

Theodosiou N, Latinopoulos P (2006) Evaluation and optimization of groundwater observation networks using the Kriging methodology. Environmental Modelling and Software 21: 991-1000.

Tizro TA, Singhal DC (1993) Geoelectrical studies in Narnaul area for estimating transmissivity of alluvial aquifers. National seminar on hydrological hazards: Prevention-Mitigation. Univ. Roorkee, India.

Tizro TA (1995) An integrated hydrogeological study of Mahendragarh district, Hryana, India. PhD Thesis submitted to department of Hydrology I.I.T. Roorkee, India.

Tizro TA (2002) Hydrogeological investigations by surface geoelectrical method in hard rock formations. Annual Geological Conference issue. Geological Society of Malaysia, Bulletin, No 45.

Tizro TA, Voudouris K (2008) Groundwater quality in the semi-arid region of the Chahardouly basin, West Iran. Hydrological Processes, Vol. 22, Issue 16: 3066-3078.

Tizro TA, Voudouris KS, Salehzade M, Mashayekhi H (2010) Hydrogeological framework and estimation of aquifer hydraulic parameters using geoelectrical data: A case study from West Iran. Hydrogeology Journal 18: 917–929.

Todd DK, Mays LW (2005) Groundwater Hydrology. John Willey Sons Inc. New York, 636 p.

Voudouris K, Panagopoulos A, Koumantakis J (2000) Multivariate statistical analysis in the assessment of Hydrochemistry of the Northern Korinthia Prefecture alluvial aquifer system (Peloponnesus, Greece). Natural Resources Research, Vol. 9, No 2: 135-146.

Voudouris K (2009) Assessing groundwater pollution risk in Sarigkiol basin, NW Greece. In: River Pollution Research Progress, Chapter 7, 265-281. Nova Science Publishers Inc. (Eds: M. Gallo and M. Herrari).

Voudouris K, Lambrakis N, Papatheodorou G, Daskalaki P (1997) An application of factor analysis for the study of the hydrogeological conditions in Plio-Pleistocene aquifers of NW Achaia (NW Peloponnesus, Greece). Mathematical Geology, Vol. 29, No 1, 43-59.

Voudouris Κ, Christodoulakos Υ, Steiakakis Ε,  Angelakis ΑΝ (2013) Hydrogeological characteristics of Hellenic aqueducts-like qanats. Water 5, 1326-1345 (Special Issue: Worldwide History of Water Supply, Sanitation, Wastewater and Stormwater Technologies).

Webster R, Oliver M (2001) Geostatistics for Environmental scientists. John Wiley & sons, Ltd.

Xu C, Gong L, Jiang T, Chen D, Singh VP (2006) Analysis of spatial distribution and temporal trend of reference evapotranspiration and pan evaporation in Changjiang (Yangtze River) catchment. Journal of Hydrology 327: 81– 93.

Zohdy AAR (1989) A new method for automatic interpretation of Schlumberger and Wenner sounding curves. Geophysics 5 (2): 245–252.

Zohdy AAR, Eaton GP, Mabey DR (1974) Application of surface Geophysics to groundwater investigations. US Geological Survey Techniques of Water Resources  Investigations, Book 2, (Chapter D1), 116 p.

Table captions

Table 1. Summary statistics of chemical parameters in groundwater samples

Table 2. The correlation coefficients between the major ions in groundwater

Table 3. Best-fitted variogram models of groundwater quality and their parameters

Table 4. Selecting the best interpolation method according to RMSE and MAE

Figure captions

Figure 1. Geological map of the study area, showing the sampling site location

Figure 2. Transmissivity (m2/h) distribution map

Figure 3. Fence diagram of study area

Figure 4. Experiment and theoretical semivariogram of EC

Figure 5. Experiment and theoretical semivariogram of pH

Figure 6. The spatial variation of pH  by using Co-Kriging method

Figure 7. The spatial variation of EC by using Co-Kriging method

Figure 8 . The spatial variation of TDS by Kriging method

Figure 9. The spatial variation of Na+ by using Co-Kriging method

Figure 10. The spatial variation of SAR by using Co-Kriging method

Table 1. Summary statistics of chemical parameters in groundwater samples

| St. Dev.* | Mean | Min | Max | Unit | Groundwater quality parameters |
| --- | --- | --- | --- | --- | --- |
| 0.195 | 7.23 | 6.85 | 7.78 | - | pH |
| 1.82 | 4.5 | 2.08 | 9.77 | meq/L | HCO3- |
| 817 | 1024 | 244 | 4040 | meq/L | TDS |
| 2.86 | 3.035 | 0.266 | 13.63 | - | SAR |
| 1335 | 1695 | 410 | 6580 | cm/Sμ | EC |
| 5.77 | 6.09 | 0.41 | 23.34 | meq/L | Na+ |
| 0.18 | 0.123 | 0.007 | 0.831 | meq/L | K+ |
| 3.53 | 4.35 | 1.01 | 16.48 | meq/L | Mg2+ |
| 5.24 | 5.96 | 1.47 | 28.42 | meq/L | Ca2+ |
| 7.9 | 6.97 | 0.37 | 34.08 | meq/L | SO42- |
| 6.57 | 4.67 | 0.165 | 27.1 | meq/L | Cl- |
| 450.45 | 511.97 | 130 | 2090 | meq/L | TH |
| 13.1 | 16.14 | 3.5 | 63.93 | meq/L | Sum Anion |
| 13.3 | 16.51 | 65.34 | 3.8 | meq/L | Sum Cation |

*St. Dev.=Standard Deviation

Table 2. The correlation coefficients between the major ions in groundwater

|  | TH | SAR | Sum Cation | K+ | Na+ | Mg2+ | Ca2+ | Sum Anion | SO42- | Cl- | HCO3- | pH | TDS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| SAR | .30* | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Sum Cations | .97** | .39** | 1 |  |  |  |  |  |  |  |  |  |  |
| K+ | .42** | 0.11 | .43** | 1 |  |  |  |  |  |  |  |  |  |
| Na+ | .84** | .49** | .94** | .38** | 1 |  |  |  |  |  |  |  |  |
| Mg2+ | .88** | .33* | .91** | .58** | .86** | 1 |  |  |  |  |  |  |  |
| Ca2+ | .95** | 0.24 | .88** | 0.25 | .70** | .68** | 1 |  |  |  |  |  |  |
| Sum Anions | .97** | .39** | 1** | .44** | .95** | .91** | .88** | 1 |  |  |  |  |  |
| SO42- | .94** | .35** | .91** | .31* | .78** | .83** | .89** | .90** | 1 |  |  |  |  |
| Cl- | .74** | .39** | .84** | .33* | .90** | .70** | .66** | .85** | .56** | 1 |  |  |  |
| HCO3- | 0.21 | -0.05 | 0.21 | .62** | 0.15 | .43** | 0.05 | 0.21 | 0.10 | 0.01 | 1 |  |  |
| pH | -.37** | -0.14 | -.35** | -0.2 | -.32* | -0.25 | -.41** | -.37** | -0.24 | -.42** | -0.12 | 1 |  |
| TDS | .97** | .40** | .99** | .43** | .94** | .91** | .88** | .99** | .92** | .83** | 0.20 | -.35** | 1 |
| EC | .95** | .41** | .99** | .43** | .94** | .90** | .86** | .99** | .91** | .83** | 0.45** | -.32* | .99** |

*Correlation is significant at the 0.05 level (2-tailed)

**Correlation is significant at the 0.01 level (2-tailed).

Table 3. Best-fitted variogram models of groundwater quality and their parameters

| R2 | Co/(Co+C) | (m) Range | Sill | Nugget | model | parameter |
| --- | --- | --- | --- | --- | --- | --- |
| 0.95 | 0.847 | 96100 | 0.040 | 0.034 | Linear | pH |
| 0.78 | 0.476 | 95950 | 3.22 | 1.53 | Exponential | HCO3- |
| 0.90 | 0.44 | 96000 | 0.61 | 0.27 | Spherical | TDS |
| 0.87 | 0.37 | 95500 | 1.29 | 0.48 | Exponential | SAR |
| 0.86 | 0.41 | 96450 | 0.58 | 0.24 | Exponential | EC |
| 0.89 | 0.28 | 10015 | 1.8 | 0. 52 | Exponential | Na+ |
| 0.87 | 0.368 | 66300 | 0.0312 | 0.0115 | Exponential | K+ |
| 0.96 | 0.55 | 59580 | 14.9 | 8.37 | Spherical | Mg2+ |
| 0.87 | 0.39 | 98100 | 0.457 | 0.179 | Exponential | Ca2+ |
| 0.7 | 0.25 | 49340 | 1.94 | 0.503 | Exponential | SO42- |
| 0.65 | 0.238 | 66200 | 2.29 | 0.544 | Exponential | Cl- |
| 0.95 | 0.425 | 97900 | 0.527 | 0.224 | Spherical | TH |
| 0.87 | 0.420 | 83100 | 0.61 | 0.26 | Exponential | Sum Anions |
| 0.87 | 0.420 | 91250 | 0.60 | 0.26 | Exponential | Sum Cations |

Table 4. Selecting the best interpolation method according to RMSE and MAE

|  | Kriging |  | Co-Kriging |  | IDW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Power 1 |  | Power 2 |  | Power 3 |  | Power 4 |  |
|  | RMSE MAE |  | RMSE MAE |  | RMSE MAE |  | RMSE MAE |  | RMSE MAE |  | RMSE MAE |  |
| pH | 0.20 | -.008 | .202 | .002 | 0.202 | 0.0039 | 0.23 | -0.006 | 0.23 | 0.009- | .236 | -0.008 |
| HCO3- | 1.812 | 0.038 | 1.79 | .041 | 1.83 | 0.069 | 1.98 | 0.069 | 2.07 | 0.078 | 2.103 | 0.085 |
| TDS | 740.3 | -18.9 | 727.8 | -17.9 | 697.1 | 61.26 | 731.4 | 118.7 | 781.5 | 144.4 | 805.8 | 155.2 |
| SAR | 2.56 | .14 | 2.46 | .18 | 2.58 | 0.136 | 2.69 | 0.25 | 2.77 | 0.27 | 2.83 | 0.28 |
| EC | 1161 | -20/85 | 1137 | -19.98 | 1141 | 97.83 | 1191 | 191.7 | 1282 | 243.7 | 1323 | 253.2 |
| Na+ | 4.81 | .53 | 4.66 | .57 | 4.7 | 0.46 | 4.7 | 0.9 | 5.08 | 1.13 | 5.27 | 1.23 |
| K+ | 0.169 | 0.004 | 0.165 | 0.005 | 0.166 | 0.0015 | 0.19 | 0.016 | 0.206 | 0.025 | 0.215 | 0.028 |
| Mg2+ | 3.18 | 0.033 | 2.91 | 0.014 | 2.803 | 0.299 | 2.66 | .54 | 2.815 | 0.63 | 2.91 | 0.66 |
| Ca2+ | 4.61 | -0.11 | 4.58 | -0.179 | 4.85 | 0.2 | 5.53 | 0.4 | 5.86 | 0.56 | 5.99 | 0.62 |
| SO42- | 6.97 | .327 | 6.73 | 0.23 | 6.88 | 0.68 | 7.15 | 1.002 | 7.58 | 1.108 | 7.77 | 1.15 |
| Cl- | 5.48 | 0.02 | 5.35 | 0.059 | 5.51 | 0.18 | 5.79 | 0.8 | 6.19 | 1.14 | 6.409 | 1.27 |
| TH | 361.9 | -14.89 | 357.3 | -14.15 | 349.8 | 25.12 | 376.4 | 48.95 | 400.8 | 59.68 | 412.1 | 64.09 |
| Sum Anions | 11.35 | -0.23 | 11.33 | -0.196 | 11.12 | 0.93 | 11.65 | 1.88 | 12.46 | 2.32 | 12.86 | 2.517 |
| Sum Cations | 11.65 | -0.21 | 11.31 | -0.24 | 11.37 | 0.97 | 11.94 | 1.91 | 12.77 | 2.35 | 13.18 | 2.54 |

Figure 1. Geological map of the study area, showing the sampling site location

Figure 2. Transmissivity (m2/day) distribution map

Figure 3. Fence diagram of study area

Figure 4. Experiment and theoretical semivariogram of EC

Figure 5. Experiment and theoretical semivariogram of pH

Figure 6. The spatial variation of pH using Co-Kriging method

Figure 7. The spatial variation of EC using Co-Kriging method

Figure 8. The spatial variation of TDS using Kriging method

Figure 9. The spatial variation of Na+ using Co-Kriging method

Figure 10. The spatial variation of SAR using Co-Kriging method